The optimizer produces λ_i(t) = A(t)·cos(ξ(t)·t + φ_i) — a sinusoid in absolute time.

But the observation (load state + drone state) doesn't contain t, ξ, A, or the phase.

So in principle the same observed state at two different times wants two different λ's → regressing obs → λ could be ill-posed.

The saving grace might be: each drone flies an elliptical orbit driven by that sinusoid,

so its own position+velocity encodes where it is in the cycle (the phase). If so, obs → λ is recoverable.

If not, IL-of-λ won't fit well and we'd need to add a phase feature (or rethink the target).

The optimizer's clock drives each drone around an elliptical orbit. On a closed orbit,

where you are on the orbit (position + velocity) is the phase. So the same motion can be produced by a law that says

"given where I am on my ellipse right now, here's the λ that continues it" — no clock required.

The phase the optimizer tracks with time is implicitly stored in the drone's own position.

And the deepest version of it: the drone's position is a physical integrator of the oscillation.

The environment itself remembers the phase for you — it's literally accumulated in the drone's coordinates,

and that coordinate is in the observation. So the "external clock" isn't really external once you look at the plant state:

it's been integrated into an observable orbital position.

can we feed a history of the drone state as the observation? instead of just the current state?

MSE = mean squared error between predicted and true λ. The reference point is the dumbest possible predictor:

ignore the observation, always output the average λ. Its MSE is exactly the variance of λ,

which here is std² ≈ 0.95² ≈ 0.90.

line 2226 in chat transcript

1. β-mixing on the rollout — and here it's free to do safely. On DAgger iteration 0 the pure policy blows up immediately,

so a naive rollout collects only deep-in-the-buzz garbage states, not the "slightly off the orbit,

needs to recover" states that actually teach recovery. Standard fix: act with λ = β·λ_expert + (1−β)·λ_policy,

β starting near 1 and decaying each iteration. Normally mixing controllers is risky,

but G·N=0 makes it risk-free here — both λs only touch the nullspace, so any blend leaves the load tracking untouched.

You stay near the good orbit while slowly letting the policy explore off it. Clean.

2. One honest catch I want to flag before we build it. Remember why the expert is stable:

its λ = A·cos(ξt + φ) is driven by the clock, not by drone position. It's phase-feedforward —

it literally can't drift because time can't drift. The policy replaced the clock with position-inference,

i.e. phase-feedback, and that's the unstable path. Now, when DAgger queries the expert at an off-orbit state at time t,

the label it returns is the clock value λ(t) — which may not be a consistent function of the observed position off-orbit

(same-ish position, different time → different label). If phase genuinely isn't recoverable from the off-orbit observation,

DAgger's regression floor is Var(λ) and it won't fully converge. It might still improve a lot;

I just don't want to promise it's a magic bullet without seeing the numbers.

- if the network learnt to mimic optimizer, which part exactly? can it only get its inputs. it was only trained to produce

one lambda from its observations, not all of them, but they are homogenous, but it doesn't matter because it just outputs

its own lambda.

>>>>  the network was trained to only produce its own lambda, but it should train to produce the whole vector,

that's what the optimizer does with just the load state, attachment points, and phases , we should be able to do the same

- problem: residual rl changes the force, changes the drone state, estimate no longer valid, so f1 part will get messed up

and f2 will have to learn to fix it as well. i mean that's also the what would've happened to the  classical controller and

optimizer because the q they rely on to calculate prdot would be wrong, making the optimizer kinda obselete because it

doesn't even see its own correct velocity , i think the reward function will have to change to account for this,

because not just the load tracking will fail, also the loitering

>>>> don't estimate f, just raw dog the whole lambda vector

-so i'm convinced this is recoverable from the kinematics alone because this is what the optimizer does, it does not look

at the drone measurements from the fmu, it just recalculates it, and therefore its lambda comes from the same guess and

the same propagation of q(kinda), the only thing the optimizer takes from the fmu is the load state, actually by looking

at the optimizer, the force it uses is even wrong because it doesn't go throught the filter, and yet it doesn't diverge.

in fact, by looking at the optimizer it uses a lot of math to determine lambda and lambda , recalculates everything

analytically unlike what we did when we take f and fdot directly

also you're saying at the reconstruct function explanation that this is what blows up, and it's also what's different

so maybe it's worth to look at it more closely

- learn the error from myself? the difference should only be the low level controller , in my model i just have a unit

delay , but i don't want to hardcode this into the code because irl it will be different

-where does the 1 cm error come from? probably the unit delay

-i want to see matrix N

- The belief filter still computes all n forces; the own slot's belief just goes unused now (harmless). why?

i thought the lambdas are the beliefs? but my lambda is known for certain because i know my position and vel ?

the force is then calculated and it the applied force, my segment of the vector at least, and the other part of the

vector for the other drone force is not needed, also i don't even see in the code where does this happen, what happens in

the code is i estimate all the positions and velocities but i throw away my belief about my state, not the force

-is it possible to do something like dagger a bit as a reward using a centralized controller that sees everything without

noise, train on that , so the residual learns to resist noise/desync , then do RL on top . basically the residual will

learn to correct the wrong calculated force to match the correct force coming from the centralized expert

-in verify reconstruction : N, DT, T_END, EPS, PHASES, LLC_ALPHA, FZ, and BYPASS_OPT=True (drive with the fast

fixed-sinusoid controller — reconstruction is controller-independent, pure kinematics). i kinda disagree, the optimizer

affect the velocity, so we would want to know if our estimate is still good when the optimizer is active, i understand

it's a force to velocity relationship, but since we have an error we don't know which forces exagerate it? also since the

thing explodes around 6.5s which is when the optimizer is alive, maybe worth looking into

>>>>>>> the last couple of points can be completely overcame by just raw dogging the whole lambda vector(not really,

still might be useful)

how does the network know just from the load state that it needs to speed up? feels very convoluted to understand this.

also i think dagger will still be needed once we generalize to multiple trajectories and so, because currently i think it

just memorized a mapping and didn't learn truly a control law

i think this is a problem, the network just memorized what the optimizer did, i think this is fundamentally ill posed,

it cannot know when to increase/decrease A and ksi (implicitly by increasing lambda amplitude and frequency)  just by

looking at the load state, either that or it will maybe just learn to loiter in a way that guarantees the vel norm never

reaches a certain value, but even then, i don't think it does that. but again, i don't see how the load state even matters?

it's just a number it uses to memorize probably(in this load state you increase A , in this one you increase ksi,

not a control law), the optimizer gets w_d as input too btw(and it's extremely crucial), which we ignored

if we think conceptually, what does the optimizer do:

it takes: the load state + problem geometry(let's start with easy case when this is just a constant), builds vLi,

builds G and N

then takes G and N + has a memory of old G and N (idk how to replicate that in an NN) builds Gdot and Ndot,

then takes w_d + has memory of old w_d, builds w_d dot, and then uses all that to Ti and f_perp_dot (aka qdot)

then uses vLi and qdot to reconstruct the drones vel norm and uses the phases to build the lambdas , changes the A and ksi

of the lambdas based on the drones vel norm

to sum upp, we need to at least add w_d, and the memory thing to calculate the derivates, idk how to encode that?

optionally we add the epsilon and the attachment points for more generalization but that's a next step

we probably need heavy handed feature engineering here, calculate all these derivatives outside the net and provide them as

inputs, i mean it's a waste anyway to force the NN to learn all this math that can be done outside super easy

The prdot DAgger has the same skeleton but three adaptations: (1) it's whole-vector — one 30-D input row and one λ[4] label

per step, not 4 per-drone rows; (2) the rollout must reconstruct ṗ_R from the applied (mixed) λ history, since that's the

state the policy actually visits — this is the exact covariate-shift the deploy buzz exposed; (3) λ̇_{t-1} from the

applied-λ history. β-mixing is still safe (G·N=0).

-how does the noise survive the filter : the λ self-feedback loop (λ_{t-1} → ṗ_R → λ_t) — the exact loop that buzzed

before DAgger — is getting re-excited by the noise. DAgger cured the covariate-shift trigger, but sensing noise is a

different trigger for the same latent instability. That would explain why it survives the LLC low-pass:

a sustained limit-cycle in λ (a few Hz) passes the filter partially, whereas pure one-off noise would get smoothed.

(might be bs)

clock features are a 14-D Fourier oscillator bank,

- F2 : while the load doesn't swing too much, it seems like the drones loitering is what suffers the most

- filter is supposed to be like the drone low level controller dynamics, so we shouldn't have access to its exact value

but a close estimate maybe

how to do RL on top of a learned policy from IL:

1 - KL penalty to the warm-start policy (the RLHF move). Keep a frozen copy π_ref = the DAgger/supervised warm-start,

and add to the reward: r_total = r_task − β · KL(π_θ(·|s) ‖ π_ref(·|s))

For Gaussian policies the KL is analytic. β is the leash length — high β = stay glued to the warm-start,

low β = free to roam. Very common to anneal it (tight early, loosen as it proves stable).

2 -  Behavior-cloning auxiliary loss. Add a supervised anchor to the policy-update loss:

L = L_PPO  +  λ_BC · E_s ‖ π_θ(s) − a_ref(s) ‖²

where a_ref is the warm-start's action (or the demonstrations). This pulls the mean back toward the warm-start each update.

This is the PPO+BC / "RL-with-demonstrations" trick, and it's simple to bolt onto MAPPO.

plan:

supervised (DAgger-style) warm-start → residual imitates the centralized force-correction.

MAPPO with the velocity-aware reward, KL/BC-anchored to (1) so exploration can't unlearn it

-include wd and f local or no? in f2 observation space

should we give wd and flocal or G*wd and N*lambda? which would be easier for the network?

or is it too difficult to tell what is useful to it

- make sure residual cap is not preventing learning

arithmetic: 18 (load) + 6 (own) + 3 (f_g) + 3 (f_λ) = 30, not 33. I mislabeled it 33 all along.

The obs is 30-D. (Amusingly the base net is also 30-D, but a different 30: clock14+ṗ_R12+λ4.)

-in f2 warm start dagger : The DAgger aggregation is walking the system into ever-worse states and training the net on

ever-more-explosive, mostly-un-achievable targets (δf* max ~5 N, but the cap only permits ~1–2 N).

So the net learns to saturate.

what is  AR(1) Gaussian?

do we output mean only? or mean with std?

I'm worried that the decoupling of the drones states from the laod state is making learning harder because the load is

stable even when the drones are struggling

-understand failure mode of the desync and build the reward around it

let me retry to rephrase this in my thinking

the problem with feeding noise and desynching , it causes the force to be noisy a bit and a bit "off" from what it's

supposed to be, (even feeding that to the classical controller would cause the same issues), even when we do regression

it doesn't fix the problem because even if the network learns a general magnitude and  direction that

"minimize the difference" , but no matter it will not be exactly the same, and once beta = 0.9 , it will start injecting

its noise into the state and will visit new states it's unable to handle properly, so again all these noises will keep

adding up and causing it to devaite and explode (basically you can't regress noise) , the only way to deal with this

is to learn a sound policy/understand some deeper law that allows you to fly smoothly

Regression asks: "predict the correct force" → impossible with hidden noise.

Control asks: "react so the system stays stable and smooth" → possible with feedback, because a damper doesn't need to

know the true state — it just opposes bad motion (high velocity, collapsing tension). You can be blind to the exact right

force and still push back on "I'm accelerating too hard" or "my cable is going slack"

That's the thing regression can't express — it's a static input→output map with no closed-loop objective —

and it's exactly what RL optimizes for (outcome: stable/smooth trajectory). But the honest bar isn't perfect

(the info loss is permanent); it's robust and bounded: keep velocity sane and tension up despite not knowing the truth,

rather than trying to cancel the noise exactly.

i don't know if warm starting helps or not in f2

split the residual delta force to two components, one next to w and one in the nullspace(internal forces)

At stall_w=300, a v=0.1 dip costs 300·(0.25−0.1)² = 6.75/step — already way bigger than tracking (~0.05).

So the policy should avoid it. Two reasons a quadratic-only floor still leaks:

The quadratic has zero gradient at the threshold. At v=0.24 the penalty is 300·0.0001 = 0.03 — basically nothing.

So it doesn't defend the boundary; it only reacts once you're already deep. The remaining dips are the policy sliding

across a soft edge.

Stall penalty is sparse (only during a dip); tracking is dense (every step). So brief dips can still be "worth it"

over an episode even at a high weight.

The fix for both is to steepen the floor with a linear hinge (constant, strong gradient right at ε),

plus a startup grace so I can crank the weight without the unavoidable spin-up-from-rest transient blowing up the reward.

New stall penalty:

Stall penalty is now stall_w·(d² + d) with d = relu(ε−v) — the linear term defends a margin above ε instead of only reacting

once deep. stall_w 300 → 400, and the first 20 steps are graced (spin-up from rest isn't a controllable stall,

so it's no longer penalized — that also lets the big weight not wreck team_ep_R). swing_w stays 0.

Foundation (the architecture that made any of it work): two-head residual [δλ nullspace + δw range],

clock in obs (44-D), time-indexed expert tracking, per-drone GAE, batch 10k→20k, deterministic held-out eval,

blowup guard, warm-start+resume. This broke the tracking-vs-load Pareto see-saw → fixed-scenario DET_loop 0.18, load calm.

Reward regimes (all warm-started):

expert-track(w=1) + stall-light(w=50, floor ε=0.25) + load(w=10) → first acceptable results (fixed 0.18; held-out ~0.28).

stall cranked 50→300 (and — the detour — we'd briefly added a swing/load-velocity term (w=10) for smoothness; it bribed

the policy into slowing down → drones stalled, so we removed it, swing_w=0). Deep stalls mostly gone → better.

stall 400 + linear hinge + margin → punish even approaching ε (cruise floor 0.30, not just below 0.25).

(planned) + jerk smoothness (state-based) + ‖Σfᵢₙₜ‖² coordination/load-neutrality term; tune down manifold_w to a

small anchor → the task-reward pivot to surpass the expert (with a small anchor to prevent the all-zero degeneracy).

The one thing your draft dropped that's worth keeping: the swing-term detour — because the lesson is "don't naively

re-add a load-velocity penalty; it fights airspeed."

Quick recap of what's logged:

Foundation — two-head split, clock, time-indexed tracking, per-drone GAE, DR, warm-start/resume, blowup guard.

1 expert + light stall(50) + load → first acceptable (0.18).

2 stall 50→300, swing removed (the detour) → mostly no stalls.

3 stall 400 + hinge + margin (punish approaching ε) ← running.

4 planned: jerk + ‖Σfᵢₙₜ‖² + tune down manifold_w. -> didn't work

IL

Dagger

The policy input (pR_dot + lambda_{t-1}) is reconstructed from the APPLIED

(mixed) lambda history — the state distribution the policy actually visits.

i understand the idea of reconstructing prdot using the mixed lambda, but feeding the last mixed lambda in the lambda t-1 input feels kinda wrong idk why, it's supposed to know its own output no?

This creates a physically impossible state. The network receives a feature vector where the kinematics ($\mathbf{p\dot{R}}$) say the cable tensions were at level $A$, but the state label ($\lambda_{t-1}$) claims they were at level $B$.

- $\lambda_{t-1}$ is a Physical State, Not Internal Memory

In reinforcement learning and imitation learning:

- $\lambda_{t-1}$ is an observation of the physical plant's state (the actual cable force distribution that acted on the system at step $t-1$).

- It is not an internal, hidden memory state of the neural network actor.

Because the true physical state of the drones and payload at step $t$ was generated by $\lambda_{\text{applied}, t-1}$, the observation $S_t$ must reflect what actually happened to the plant.

We don’t want to show the network it’s pure ugly output from the get go

Observation space for f1, clock features

Why not just t, well I was worried the network will use it to memorize actions vs time directly(in the beginning I was using just one trajectory)

Saw tooth time(mod t) didn’t occur to me because I was looking at lambda and lambda dot so using sin and cos was the obvious choice

Training in DAgger

Two big inefficiencies

1 – net is started from random weights, this is wasting a lot of epochs to relearn stuff that can already exist in the old net -> warm starting

This might introduce some problems

### The Normalization Shift Problem (The Biggest Issue)

At each iteration $k$, newly visited off-manifold states change the global mean ($\mu_k$) and standard deviation ($\sigma_k$) of the dataset:

$$X_n^{(k)} = \frac{X - \mu_k}{\sigma_k + \epsilon}$$

The weights of the first layer ($W_1$) in the previous network were optimized to process inputs normalized by $(\mu_{k-1}, \sigma_{k-1})$.

When you update the normalization parameters, every feature entering Layer 1 shifts suddenly. If you reuse the old weights $W_{k-1}$, those initial layers receive activations they were never calibrated for. Warm-starting on shifted inputs often leads to erratic gradient spikes during early fine-tuning epochs.

The solution to this is to either eat the cost, whatever, train for an extra epoch or two, still better than random weights or scale the first layer weights using the old and new mean and std

### The Local Minima Trap & Sample Imbalance

By iteration 5 or 6, the old dataset $D_{\text{old}}$ (e.g., 50,000 samples) heavily outweighs the newly added rollout data $\Delta D$ (e.g., 2,500 samples).

you don’t learn much from the new data, especially toward the latter DAgger iterations which matter the most, this still happens anyway when training from scratch but I guess warm starting makes it even worse(esp with a small learning rate, I get stuck in the same local minima. With a big learning rate I kinda destroy the features I already learnt and it becomes hard to fine tune anyway, so maybe a dynamic learning rate becomes needed)

OR

This leads to the next point, this method of combining data is problematic but also inefficient

2- Dataset size explodes towards the end, by the last couple of iterations the dataset size is 10x the original size, this is both problematic as discussed before and inefficient because we need to train on too much data that’s redundant -> train only on new data or a mix of sampled old data and new data

Training on new data only might cause catastrophic forgetting -> counter point: we don’t care or even we want that to happen

the states that actually matter for fixing covariate shift are the ones near beta≈0, where the policy is driving and visiting states the original expert-only collection never saw. Those are exactly the hard, informative, off-manifold corrections DAgger exists to generate. Weighting all iterations equally means the redundant near-duplicate early data dilutes the training signal from the harder, more valuable late-round data.

- Early Iterations ($\beta = 0.9$): The expert keeps the system on a tight leash. The data generated here reflects the expert's distribution $P(s \mid \pi_{\text{expert}})$, not the policy's failure modes. You don't care as much about keeping this data long-term because it doesn't represent autonomous execution.

- Late Iterations ($\beta = 0.0$): The policy runs autonomously. If it stays near the nominal trajectory, the data collected at $\beta = 0$ implicitly includes both nominal states AND realistic execution drift.

Because the rollouts at $\beta = 0$ naturally contain nominal tracking states alongside its own micro-corrections, over-indexing on recent $\beta = 0$ data (like an 80/20 split or a aggressive rolling buffer) directly optimizes the policy on its actual deployment manifold.

there's value to reducing the dataset size especially when generalizing, i don't care if the net forgets a bit the old data(in fact we might say the old data is only important for warm starting and not helpful for the final trained policy), it was generated with more help from the expert that won't exist in the future, as long as it does well on the new data then it's good, to clarify my point, the last two iterations are closed loop control of the policy on its own, if it does well there, then we don't need to worry about anything else, but even if, let's say we want to keep some some of the old data. For reasons that it shows the net how to recover from really bad states that it won’t generate when it’s fully trained, If for some reason it visits them somehow, then it makes sense to only keep part of the old data for example 20% old data + 80% new data(all the new data but it will be 80% of the total dataset size)

The question then becomes how to choose the 20% old data

Uniform: Uniform random subsampling of the retained 20% will, by chance, mostly keep "boring" steady-state loitering rows (since that's most of any episode) and only occasionally keep the rarer difficult maneuver rows. Not good especially towards the end

Recency bias: sample more from iterations that are more recent

Hardness bias: if a data point is considered “easy” e.g. the policy output and expert output are close either entirely dismiss it or give it much less weight than “hard” points during sampling the “old data” in the dataset

Stratifying and binning: guarantee diversity by classifying points based on maneuver location (loitering, turning, moving in a straight line...etc. Done by reference velocity and acceleration) and take a certain percentage from each bin, the percentage can be determined by a hardness score for the bin for example

The last two ideas especially can be mixed in different ways, for example even the data points in the bins can be selected based on difficulty to ensure good representation

One final point: All these suggestions add complexity that must be justified by the failure of the simplest solution. Retraining from scratch requires zero hyperparameter tuning.

Each DAgger iteration generates new data(we label it with hardness, age, and bin/category) 120K rows, we keep all this data to train on, additionally, we augment it with uniformly sampled old data from a pool(while respecting minimum per pool count, if a pool gets less than a minimum 20% we select more until this limit is guaranteed) (around 65% of its size, so final split is 60% new , 40% old, 80 old, total 200k) , after the training is done, the new data (120k) is added to the old pool and then the some data gets uniformly sampled(same minumum % per bin idea) to be evicted from the pool (pool size is 250k)

if a certain dagger iteration evaluation proved to be not satisfactory (metrics in how good is it at imitating the optimizer), the iteration is repeated (same beta) and the training and pool data size are allowed to expand without any sampling (so all data is used in training and all data are kept in the pool). once its performance becomes good , we shrink back to normal sizes

the bins are

- 0 = loiter — reference speed < V_LO=0.05 m/s (parked)

- 2 = transition — reference |accel| > A_HI=0.02 m/s² (ramping)

- 1 = cruise — everything else (steady motion)

Applying the filter without including it in the training is strange, like we try to minimize MSE in training and then we say ok we’re not gonna apply that actually we’re gonna filter it, in my mind MSE should be on the final output, which includes the filter. But yea it helps the net a lot. I would like to understand why and how it works

1 - A and ksi are non decreasing yes, that's intentional, but it's not true that every once in a while prdot hits 0 , not true at all, i can see their plots, in this graph they don't get close to 0 at all, they should only go up when it's needed, i see they're going up all the time

2 - the clock features are absolutely necessary , the load and drone velocity is not enough because during loitering we can have the same load position and drone vel but different lambda, it needs a way to access time / periodicity without learning to memorize

i have many frequencies because ksi can change from trajectory to another and within the same trajectory so it needs to learn to switch to a higher frequency and time itself using it but only conditioned on prdot hitting epsilon, the idea was, if in some trajectories A and ksi change during the trajectory based on prdot, it will learn to copy that, if in some other trajectories A and ksi remain constant, then it will be forced to learn the dependency on prdot not on time, to further protect, we might look into adding trajectories that extend beyond any time marking, i think currently we mark until t = 25? we can use episodes of 30 or 35 seconds

need to show math here proving the 25 seconds

clock frequencies are arange(1.5, 3.0, 0.25) = {1.5, 1.75, …, 3.0} — all integer multiples of 0.25 rad/s. So the joint sin/cos vector is periodic with fundamental frequency = gcd = 0.25 rad/s, i.e. it repeats exactly every 2π/0.25 = 8π ≈ 25.13 s.

So the clock is a unique positional code only on [0, 25.13s); the tail [25.13, 35] (~9.9s) is a verbatim replay of [0, 9.9]. The net physically cannot use the clock to tell t=26 from t=0.9 — to emit the right λ there it must read pR_dot/prev_λ

You've turned the last ~10s of every episode into a forced no-memorization zone, for free, just by setting T_END past the clock's period.

Two things that make this actually bite (and one caveat):

It only matters where the tail's correct λ differs from its aliased-early twin — i.e. where there's motion in the tail. Which is exactly why ensuring movement to ~t=30 (customs to 30, quintic move-end U[20,30]) matters: a pure-loiter tail would alias to a near-identical λ, so the ambiguity would be benign. The two design choices reinforce each other.

Net picture: randomized timing guards [0, 25.13s], the clock's own repetition guards [25.13, 35s] — two independent anti-memorization pressures, and the "movement to ~t=30" choice is what makes the second one actually engage. Nice property of the T_END=35 pick. so the tail lands in the aliased zone by design.

ok so the plan now is to add a bunch of other trajectories that will engage the solver, maybe same ideas as this trajectory but movement in the y, z and combination of x y and z , the thing is, if it's one trajectory i'm scared it will drown between 50 trajectories , maybe having 5 of them would be ok, 5 out of 55(50+5) is significant to force the network to learn i guess , also we will save alot of time because we engage the optimizer so little (nice!) , but we will have to retrain, anyway, last thing, we need to make the episode length like 35 seconds, and we need to make sure that there is movement up to 35 seconds not just pure loitering everytime, so in the 4/5 "custom(not quintic)" trajectories we will add/have let's make the ramp up until t = 30? also we can increase the quintic trajectories position range, so that we have movement until more than 15s(also maybe up to 30 or so, or maybe make it random from 20 to 30, just to make sure it's not memorizing

+5 custom (non-quintic) solver-engaging trajectories — default-style straight moves but in y, z, and xyz combos; 5 of ~55 so they don't drown but force the net to learn adaptive A/ξ.

T_END 25 → 35 s, with movement lasting to ~t=30 on the custom ones (no pure-loiter tail to memorize).

Quintics: raise POS_RANGE and randomize move-end ~U(20,30)s so there's no fixed "move ends at 15" marker.

Retrain: re-collect (fast now via the deadband) → BC → DAgger(0.9→0, in-loop EMA).

-curate functions sucks for many reasons:

1- it's stochastic, meaning we can easily discard hard points and keep easy points, unlikely but still happens, not cool

2- more importantly, it assigns a survival probability per data point and each new iter we have a different data size  , that's now what i wanted at all. i wanted to say ok, we're gonna use 100k points of old data for example + the new data gives us idk 50k points

for the old data we put it in bins, stratify the bins based on difficulty, then say we take 20% of the 100k points from bin 1, 50% from bin 2, 30% from bin 3,

ok inside the bins we can assign probability of survival based on difficulty and recency(but they should be separate filters!!) , what you do is mix and mush everything together , the bin, the hardness, the recency

but they should be separate, i want a guaranteed 20% from this bin, i then want a guaranted 80% from the harder points in this bin, i then want a guaranteed 50% from more recent ones

idk if your math leads to the same results on large numbers, but i don't like that we end up with a different number each iteration and we have no guarantees on the content, like a recent easy data point has the same probabilty like an old hard one, that shouldn't be the case, it should be : the hardest and most recent 100k points but with some guaranteed minimum diversity, anyway, no matter , we turned it off, but i still want to clean it and give it a go eventually,

also cap with recency priority? absolutely not, hardest wins! that's why we struggled a lot with bad data, we were not training on the hard samples !         H_rows.append(float(np.linalg.norm(lam - prev_lam))) ?? lambda and previous lambda???

why are you curating in train prdot?? the point of curating is to prevent data explosion by choosing from the OLD data, this is the first dataset, no need to curate

one last thing, how many data points do we collect in a single iteration now? to make the cap realistic

another monestrosity that caused very very long training, we add the new data to the old data and then mask!!! no! we need to train on the new data FULLY, and sample some of the old data to remain below the total allowed data

i accidently warmstarted a fresh dagger attempt (from beta = 0.9) with an old net that was trained until 0.08 or something, and it was doing much worse than the one started from the plain BC train prdot net. this means that warm starting can actually hurt the net if it learnt trash in the last iterations, this leads to the observation i have that once the net starts outputing jitter on one iteration it becomes really hard to somehow steer it back to good results, this is especially strange because i used to get like a buzz spike mid dagger run (beta = 0.6 for example) but then by the next iteration it would be back to normal, but somehow when i started the net from 0.9 with some trash it was unable to unlearn it until 0.6, then i just stopped it

A mid-run spike at β=0.6 is a shallow excursion — the net was trained cleanly through 0.9→0.7, so its weights are still mostly in the good basin. One iteration's transient perturbs it; the next iteration's still-half-expert data pulls it back. Shallow → restoring.

A net trained all the way to 0.08 into jitter has consolidated the jitter-producing features over many iterations — deep in the bad basin. Deep → stuck.

So it's not that jitter is always sticky; it's sticky once it's consolidated in weight space, and warm-start is the transmission mechanism.

training does have a restoring force toward lower buzz via lower MSE. Not blind — just not a perfect proxy.

MSE tracks buzz well, so from a good basin (shallow spike) the clean-ish data's MSE minimum sits near the low-buzz net → it recovers.

From a deep trash net, two things bite: (a) you start in a bad basin, and (b) the trash net's own rollouts poison the data (even at β=0.9 the 0.1·policy + feedback corrupts visited states), so the MSE minimum on that poisoned data is a compromise that still buzzes. The non-100% part of the correlation is exactly the slack where a jittery-but-lowish-MSE solution can live — and warm-start keeps you parked there.

Somehow the action space has a local minimum that buzzes that the network has a very hard time escaping, and training on low beta data walks the net into that local minimum and then it becomes very hard to get out. The BC trained net even tho it’s more buzzy in closed loop it’s not stuck near that local minimum

Idea: if after a dagger iteration the buzz increases, don’t warm start from that net, warm start from the net before?

Problem: we lose valuable learnings from this iter and we move to a harder one solution: we keep all of its data? Also we can retrain on that beta again, without skipping it and moving to harder betas until buzz is acceptable, warm start from last good net. the pool/training budget bloat is temporary and per-stuck-episode

it's not learning well, esp the transition parts, it has no extra trouble working with long vs short , would be interesting to run deploy prdot with tend = 60 or something

three observations, one i'm not so confident about but worth flagging, and the others i'm very confident about

let's start with the certain ones

1 - it was doing well until beta = 0.06, it had to do one retry but it improved in it, it got completely wacked in beta = 0.04, i'm talking a 2 order of magnitude explosion in the buzz and mse , idk if it's worth it to go slower their or what

2 - at some point the policy goes through a buzz phase and is usually able to recover from it later in the episode, but it recovers phase shifted(compared to expert) and remains like that until the end of the episode ,it kinda falls into this solution (local minimum) and is never able to recover from it, why specifically at beta = 0.04? idk, but also it can fall into in before that , sometimes at 0.1 or 0.08

the observation i'm not sure about

3 - there seems to be certain times where this struggle/buzz happens ,  and i think it's not entirely related to the trajectory, it feels kinda constant, for example between 20 and 25s in a lot of trajectories, maybe because there's a transition there in most trajectories so the net kinda memorizes that but applies to other trajectories where that time is not special in any way

some thoughts

1 - there's a big problem in how i'm formulating the problem the net needs to solve , when the optimizer is engaged , prdot is below a threshold, A and ksi get pushed up and remain there , first of all, the data points when this happens are soo sparse, something like 5% out of 10% of the trajectories, so less than 1% of the total data, the net cannot realistically learn from that

additionally , even if we assume it can learn from such sparse data , what will it learn, A and ksi go up by a step so lambda is a bit bigger than what it's supposed to be, okay let's assume we're able to capture such a small change, how can it remember it? it has no memory of A and ksi, a couple of steps later A and ksi are stable again at a higher point but the net has no way of knowing, prdot is on average maybe higher , but the net doesn't see average, it has to learn from very similar states that A and ksi now increased , even tho almost everything is the same , what changed was the optimizer internal states, ok this changes prdot and maybe lambda(t-1) slightly, but not by too much , it kinda  makes sense that's it's struggling a lot to learn

at the same time, i don' want to brush off that it was able to learn very well up until beta 0.06, and why does it buzz and then become phase shifted , because they should also offer hints to what's working and what's not

The phase is set by the feedback (λ_{t-1}), not by the clock. If the net determined phase purely from the clock (monotonic absolute time), it couldn't slip — there's no other clock-consistent phase. The fact that it slips by ~π and stays there means the loop has (at least) two stable phase-locks, and the net is keying phase off its own λ_{t-1} feedback, which admits both.

A memory of lambda amplifies the learning signal making learning possible on more than 1% of the data. Additional idea, add an output of A and ksi, force the net to learn to calculate them, now it has to explicitly learn them and gets punished for getting them wrong, we can also then self feed them as inputs. turns "recognize a hidden state from a 1-step cue" into "integrate a rare update" — vastly better conditioned.

Net outputs [λ(4), A, ξ], loss = MSE(λ) + w·MSE(A,ξ). λ stays free (RL-friendly); the A/ξ head only shapes the shared representation at training time. The reason it can help the primary λ output: right now the net is free to emit a λ that's locally plausible but globally phase-incoherent (which is exactly what the slip is). Forcing it to also name a single coherent (A, ξ) pressures its internal representation toward "λ is one clean sinusoid with these params" rather than a free-form curve — so the free λ it emits should come out more phase coherent.

Net outputs [λ(4), A, ξ], loss = MSE(λ) + w·MSE(A,ξ). λ stays free (RL-friendly); the A/ξ head only shapes the shared representation at training time. The reason it can help the primary λ output: right now the net is free to emit a λ that's locally plausible but globally phase-incoherent (which is exactly what the slip is). Forcing it to also name a single coherent (A, ξ) pressures its internal representation toward "λ is one clean sinusoid with these params" rather than a free-form curve — so the free λ it emits should come out more phase loss. That's lever-1 (memory) in its cleanest form, and it composes perfectly with keeping λ free: free λ output + supervised, self-fed A/ξ latent state. The aux loss keeps the fed-back state honest; the memory gives the phase-inertia that resists the slip.

Aux head only (cheap: act_dim 4→6, add w·MSE(A,ξ), record agent.prev_x as labels). Get the diagnostic + see if λ coherence improves on its own.

If aux MSE is low but the slip persists → add the self-feed (predicted A/ξ back into the input), turning the observable-but-not-remembered state into actual memory.

And it stays fully RL-compatible either way — the A/ξ head is auxiliary, you deploy the λ output.

Another idea, instead of the last 10 lambdas , use a (kinda) geometric series to cover more time while still providing momentum close to the current lambda. Log-spaced point taps (λ at lags 1,2,4,…,256 steps): each tap keeps the raw value, so it carries phase at every lag. Good for a slip anchor (a tap ~1.5s back still says "I was in-phase then"). The further away lambdas help the net understand the phase and frequency much better

Hardness bias in the data propagation across iterations is also a huge help here

Training during F1 using an identical reconstruct to the optimizer, but during f2 I reconstruct using a filter because I know I’m getting noisy readings

F2

# 29. The entire step() in one picture

┌───────────────────────────────────────────────────────────┐

│ 1. LOCAL REPLICAS                                          │

│                                                           │

│ Each drone uses its own noisy/delayed estimate            │

│ → prepare local controller quantities                     │

│ → network produces lambda vector                          │

└──────────────────────────┬────────────────────────────────┘

↓

┌───────────────────────────────────────────────────────────┐

│ 2. RESIDUAL RL                                              │

│                                                           │

│ RL chooses Δλ and Δw                                      │

│ → N Δλ = internal/load-neutral force                      │

│ → G⁺ Δw = load-space correction                           │

│ → construct final force                                   │

└──────────────────────────┬────────────────────────────────┘

↓

┌───────────────────────────────────────────────────────────┐

│ 3. ACTUATION NOISE                                         │

│                                                           │

│ Optional force noise                                      │

└──────────────────────────┬────────────────────────────────┘

↓

┌───────────────────────────────────────────────────────────┐

│ 4. PHYSICS                                                 │

│                                                           │

│ LLC filter → force derivative → plant.step()              │

│                                                           │

│ → TRUE physical state obs42                               │

└──────────────────────────┬────────────────────────────────┘

↓

┌───────────────────────────────────────────────────────────┐

│ 4b. SAFETY                                                 │

│                                                           │

│ NaN? velocity explosion?                                  │

│ → truncate safely if bad                                  │

└──────────────────────────┬────────────────────────────────┘

↓

┌───────────────────────────────────────────────────────────┐

│ 5. REWARD                                                  │

│                                                           │

│ TRUE state only                                           │

│                                                           │

│ tracking + stall + load + swing + coordination + jerk     │

│                                                           │

│ → reward                                                   │

└──────────────────────────┬────────────────────────────────┘

↓

┌───────────────────────────────────────────────────────────┐

│ 6. OBSERVATION                                             │

│                                                           │

│ TRUE state → delay + AR(1) noise → local estimates         │

│ → build each drone's observation                          │

│                                                           │

│ → return to RL algorithm                                  │

└───────────────────────────────────────────────────────────┘

The goal of the thesis is to answer a question, is it possible to have a fully decentralized policy that is able to coordinate the carrier trajectories even when the drones don’t fully 100% agree on the state of the system, can they still learn a reactive policy that is able to stabilize the load and maintain their coordinated trajectories based on their own local and noisy observations instead of relying on a centralized controller that has access to the full and true state.

The most straightforward way to test this idea is to build upon the current controller that exists instead of forcing the policy to learn everything from scratch, we already have a controller that can fly and generate trajectories in a centralized fashion, we just need to make the policy learn to correct and augment its behavior under some loss of information (mainly f2). Additionally, the f1 net also internalized the periodic nature of the loitering trajectory without having to rely explicitly on the absolute time (so in a way we can argue it has understood the underlying task and is able to reactively deal with it better?)

It’s not just about a noisy load state, what’s kinda hidden, is that with the load state and wrench command, the controller used to be able to calculate the drone velocities (using kinematics) with high accuracy. So now under this noisy and delayed load state, this calculation becomes completely unreliable and the policy is forced to learn its action solely based on that noisy load state and its own state, the other drone states are practically inaccessible (and I don’t even do the math and feed them to the f2 policy because I know they are completely unreliable, they rely on the load state and the full force and its derivative (which we don’t have locally) so it’s going to be much more unreliable than the load state)

# Why does the critic get the delays?

This is one of the more sophisticated details in your code.

The actor doesn't know the true delay scenario.

Suppose:

d=[1,2,2,1]

versus:

d=[2,1,1,2]

The same-looking local observations could correspond to different underlying difficulty.

The critic gets:

(st​,dt​)

so it can estimate:

V(st​,dt​)

rather than merely:

V(st​)

This allows the critic to say:

"Given this exact physical state and this particular delay configuration, the expected future reward is X."

MAPPO Design choices:

4 different rewards based on individual metrics and group metrics weighted and combined

Critic gets the full state + delays -> one value per time step, critic target is summation/mean of rewards + Vt

Delays are in a way part of the state, since they can make the problem harder or easier, thus changing the expected reward, giving it to the critic helps making the advantage function more accurate, as if given the current system state the action was worse than expected, it could be because of the delay and thus we should not particularly say “oh that was a bad action”

Aka The critic needs to predict the value conditional on difficulty

The critic doesn't make the action "not bad" simply because there was a delay.

Rather, it asks:

Was the outcome worse or better than what we should expect under this particular scenario? If the hard delay makes the expected return low, then a low return isn't automatically evidence that the action was bad. But if the return is even lower than what the critic expected given that hard delay, only then the advantage would be negative and the actor should learn that the action was poor.

4 separate advantages

So conceptually:

Ati​=GAE(rti​,V(st​))

for every drone i.

This means the critic is saying:

"Given the complete global situation, how good is the team's future?"

while each agent's reward says:

"How much did my contribution to the team improve things?"

should i break the state correlation when sampling the minibatch and randomly select the agent like i do with time? or should i keep the 4 agents together and just randomly sample only the time?

-doesn't summing log action probabilities contaminate it? like one action can be likely and the other one no, and PPO sees just average prob, it doesn't know which to increase and which to decrease?? same for entropy(but okay maybe for entropy this is balanced by the PPO objective?)

Both have the same answer, The loss is computed from the sum, but differentiation distributes automatically:

∂∂μi(log⁡p1+log⁡p2)=∂log⁡p1∂μi+∂log⁡p2∂μi\frac{\partial}{\partial \mu_i}\left(\log p_1+\log p_2\right)=\frac{\partial \log p_1}{\partial \mu_i}+\frac{\partial \log p_2}{\partial \mu_i}∂μi​∂​(logp1​+logp2​)=∂μi​∂logp1​​+∂μi​∂logp2​​

Since μ₁ only affects p₁:

∂log⁡p2∂μ1=0\frac{\partial \log p_2}{\partial \mu_1}=0∂μ1​∂logp2​​=0

So the gradient becomes:

∂∂μ1=∂log⁡p1∂μ1\frac{\partial}{\partial \mu_1}=\frac{\partial \log p_1}{\partial \mu_1}∂μ1​∂​=∂μ1​∂logp1​​

and similarly for dimension 2.

Nothing gets mixed. The scalar loss is just the mathematically correct objective; autograd still sends separate gradients to each action output.

The loss is one scalar, but each μᵢ only receives gradients through its own log-probability path.

- When would summing be wrong?

If the action dimensions were not independent.

For example, if the actor produced a full covariance Gaussian:

N(μ,Σ)\mathcal N(\mu,\Sigma)N(μ,Σ)

then you cannot compute the joint log-probability by summing independent terms. You'd need the multivariate Gaussian log-density, which includes covariance terms.

Your implementation uses diagonal covariance, so the sum is exactly correct.

- each drone should know somehow itself, otherwise how does it decide which slice of the force is its, like each drone calculates the full force vector, how does it know which slice to actually apply? it has to know its own index somehow? or am i missing something? and if so, then it can only learn to ouput the delta lambdas that it needs to calculate its own slice, because i think only 2 are needed, the other two can be automatically 0 , i mean that net should figure it out on its own, that outputting something there is useless and has no effect, but it will just make the work easier i think? but regardless, it's a major point to think about, how does the drone know which slice to apply? it has no way of knowing right? so we can say in the beginning the drones talk once and assign IDs or something?

-Information that is communicated between the drones once in the beginning, time: clocks are synchronized, initial A and ksi : to create the –t lambdas, Each drone ID/order: just to know which slice of the force it should apply once it calculated the full vector (it might be not necessary in deployment actually since the net can understand the id from the relative position, but during training it makes life easier until the net knows). They also all need to know the attachment points geometry.

Decomposition: desync hurts the base 0.013 → 1.102 (+1.089 m — a catastrophe), and the residual claws back +0.75 (69–72% of the gap).

i don't understand why the load error gets hurt so much by our RL , even when i disable dw, still it gets hurt , however without any RL the load looks fine i don't understand how can the load remain so stable when the drones are losing their minds even tho the noise and the delay are in the load state why are the drones internal force distribution affected while the wrench is not?

the f1 base is conditioned to output lambda based on last lambdas and the clock, it has a dependance on prdot but it's week and only gets activated once prdot hits the limit, otherwise it's gonna hold

now G is different from drone to drone , so there should be a leak, and there is, but it's only because G disagreement, what RL makes worse is it makes the lambdas themselves different and not balanced, it has no physical understanding of the lambda coordination that was happening by default because of the identical base that's gonna hold the same lambdas, now the reason the drone trajectories lose their mind is that over time, lambdas are gonna drift, and i see this in the plots, it actually starts okayish and over time loses its mind, this is because the lambdas desync, what i don't understand is how that buildup doesn't also hurt the load trajectory

The base runs the identical net on all drones → they hold agreeing λ → the nullspace forces nearly cancel → the base's only leak is G-disagreement (different load-pose estimates → different geometry). That's the 0.036.

RL doesn't push harder — it pushes misaligned. Per-drone dλ has no notion of the cross-drone λ-balance the identical base maintained for free, so it breaks the λ-agreement. The extra leak (0.036 → 0.086) is from misalignment

The load feels forces; the trajectories are the integrals of those forces. So:

Load ← bounded (capped) leak, through a regulated PID loop → stays bounded (0.086).

Trajectories ← unbounded integral of the (bounded) misalignment, through no loop → drift without limit.

Paper by tu delft indicates history not very helpful(they still used stacked history obs), zero load desync and yet their tracking sucked

Crazy marl used relative positions of other drones in observation

Why Simulink not isaac gym or whatever

Cable-suspended multibody dynamics are easy to get subtly wrong hand-coded. Simulink models them correctly out of the box.

You needed a trustworthy ground-truth plant to validate the classical controller, the CasADi optimizer, F1 IL, DAgger — none of which needed speed, all of which needed a plant you could believe. A fast-but-unvalidated numpy sim early would have meant debugging your controller against a possibly-wrong plant.

It let you move fast on the actual research (control, IL, RL) instead of spending your first months debugging rigid-body dynamics. It probably accelerated 90% of the project.

IL (phase 1 and generalization) proved to be fully doable with the fmu, first phase RL on limited trajectories also turned out to be doable, so the bet kinda paid off because I was able to reach these results faster

methods insight for your thesis: high-fidelity sims are the right choice for validation, but RL-at-scale needs a fast path validated against them — ideally you build both, in that order.

Here's the move. The classical pipeline is KF (estimator) + decentralized controller. But we already established the decisive fact: the KF only recovers the observable range(G) load channel. The coordination failure lives in the unobservable null(G) nullspace, and once you're off the manifold a better load estimate doesn't fix it. So

The interesting, unsolved part — decentralized coordination under async partial observability, without comms — is not a filtering problem. It's a decentralized decision-under-uncertainty problem. That's inherently RL/MARL. A KF is the right tool only for the easy, observable sub-problem.

That's your thesis positioning, and it's bulletproof because it's true. When the committee asks "why not KF?", your answer is: "KF handles the observable load channel — a solved sub-problem orthogonal to our claim. The coordination failure is in the unobservable nullspace, a partial-observability decision problem KF does not address. That's the RL contribution." You're not dodging KF; you're scoping it out as the easy part. Respectful and keeps RL central.

find the decentralized policy that, under the shared desync uncertainty, is self-consistent and stays on the manifold, so the unobservable drift is never triggered

decentralized cooperative manipulation as a self-modeling / equilibrium problem under asynchronous partial observability — the agents exploit policy homogeneity to predict each other's observable contribution and learn a self-consistent strategy that avoids exciting the unobservable coordination drift, all without communication

The observability analysis (range(G) vs null(G)) becomes the justification for why it's a policy problem, not a filtering one.

it's each drone's camera reading an AprilTag on a fast-moving load from its own viewpoint. That's why it's AR(1) with corr 0.995: a slowly-drifting, bias-like, viewpoint-dependent pose-estimation error, non-white and non-stationary. Important to look if this is accurate representation of the reality

- Random delay → represents asynchronous measurements. Reasonable.

- AR(1) Gaussian error → represents temporally correlated estimation error. Reasonable as a simplified stochastic model, but not a claim about the detailed physics of AprilTag detection.

The literature supports using correlated/Gauss–Markov measurement-error models rather than automatically assuming white noise. For example, work on measurement-noise modeling explicitly argues that white noise can fail to represent real measurements and treats noise intensity and correlation time as model parameters.

"We use a first-order Gauss–Markov process as a phenomenological approximation for temporally correlated measurement errors. The correlation time is treated as a simulation parameter rather than an experimentally identified property of the camera/AprilTag system.

And there is experimental evidence that fiducial-marker localization accuracy changes with relative velocity, distance, and viewing angle, rather than behaving as a fixed isotropic Gaussian error. In particular, a study evaluating AprilTags during motion found their localization accuracy varied under different relative-motion conditions

We adopt a first-order Gauss–Markov measurement-error model as a tractable stochastic approximation of temporally correlated visual measurements

# Paper: Split Covariance Intersection Filter Based Visual Localization With Accurate AprilTag Map For Warehouse Robot Navigation

The parameters are nominal, order-of-magnitude assumptions used to evaluate the robustness of the proposed method. Their calibration against real sensor data is outside the scope of this study

KF on the load state doesn’t help, it actually makes it worse

Feeding reference load instead of actual still fails, even when only fed to the lambda calculations

measured w_d → load tracks, but drones scatter(how? Because sum of noises is unbiased, but single noise is biased, very crude unscientific but captures the idea) → loop bad (this probe)

reference w_d → drones agree, but load runs open-loop → load blows up (previous probe)

Yes — the divergent G⁺w leaks into null(G): each drone applies its own (G⁺ᵢ w_d,i)ᵢ. If all four w_d,i agreed, the assembled 12-vector would be exactly G⁺w — which is pure range (orthogonal to null(G), zero null component). But because the w_d,i disagree, the assembled force is a patchwork that is not G⁺(anything) — so it has a nonzero null(G) component: F_null = (I − G⁺G)F ≠ 0. That null component is a real internal force that:

is invisible to the load (null(G) → zero wrench → load fine)

we were looking at the math at face value and forgetting this whole math falls apart once there's disagreement, lambda is not the whole nullspace anymore and the reason why the policy learnt to use dlambda is an attempt to cancel the forces that leaked into the nullspace using it , and it learnt to use some dw i guess to attempt to learn some agreement

still confused why did the RL not learn to use them properly but ok

also big coord would be good then, because it's cancelling out the leak, but then it's not symmetrical across the drones so it's counter leaking into the load wrench subspace

coord w and leak w are bad metrics, that’s why they are low but the drones are flying everywhere. They are much better at measuring counter leak in the load wrench space, but even then not perfect, because there are genuine null space forces being cancelled

why RL alone has a hard time learning this 
Easier, more direct gradients capture the heads first. Gradient descent takes the steepest path, and there are two much steeper ones:

dλ is a direct lever on loop. It moves drone position — which is the thing the loop error measures. So the "reduce loop" gradient flows most strongly into dλ (fix the symptom directly) rather than dw (fix the cause indirectly). The reward can't tell "patched the symptom" from "fixed the cause" — both lower loop this step, and dλ has the stronger immediate gradient. So RL learns the whack-a-mole.

dw already has a clean local job: load-trim. load_w directly rewards dw for reducing load error, a job dw can do alone with an observable, low-variance gradient. So dw gets captured by the easy load-trim use, not the hard coordination use. (That's exactly what diagnose_f2 showed — dw barely clipped, used for load, tracking carried by dλ.)

Coordination-entanglement kills the per-agent gradient. This is the deep one, and it's why cooperative MARL is hard. Agreement is a joint action. If drone 1 alone shifts its w_d toward the truth while 2/3/4 stay noisy, that lone correction is asymmetric → it creates its own range/null imbalance → the loop barely improves (or worsens). So drone 1, holding the others fixed (which is what its policy gradient effectively assumes), sees ~no advantage for correcting → no gradient toward it. The un-coordinated state is a local equilibrium: the payoff only materializes if all four move together, and , and independent policy gradients can't discover simultaneous-move equilibria. Each agent greedily improving its own action doesn't get there.

what's the role of RL then, that no matter how good the estimate of wd, it's not gonna be perfect, learn to find a good balance of these rewards given the uncertainity

clean

classical estimation hits a wall (KF dead-end), supervised consistency scaffolds the coordination that decentralized RL can't discover, and RL's unique contribution is the hedged control under the residual, unresolvable disagreement that neither estimation nor imitation can handle. RL isn't demoted here — it's finally pointed at the thing it's actually good at, instead of being asked to also solve a coordination-discovery problem it structurally can't.

Here's the framing that survives all of it — and it's stronger, not weaker:

The sub-laws are individually classical but COUPLED and in mutual tension, over a distribution.

Loiter feedback regulates scatter → but pulls toward load-pose-dependent (noisy, divergent) targets → drones fight → reintroduces leak. Scatter-vs-leak, same wall, relocated.

Feedforward coordinates → loses tracking.

And the hard stall constraint (fixed-wing min airspeed) couples nastily to all of it — a classical law that regulates loiter can drive a drone below stall.

All of this across 56 trajectories × noise realizations, where fixed hand-tuned gains don't transfer.

An explicit state estimator was not employed to reconstruct the load state from the available observations. The load measurements are affected by an unknown but bounded delay and temporally correlated noise. Designing an explicit observer would therefore require additional assumptions and modeling of the observation process, including the delayed measurements and correlated noise. Instead, the proposed approach investigates whether the policy can learn to extract the information required for control directly from the observations, without requiring an intermediate estimate of the full system state.

But your fundamental justification should be that explicit state estimation is not necessary for the control objective and introduces additional modeling assumptions that you don't want to impose.

A known decentralized cooperative controller is robust under ideal sensing but becomes unstable/degrades severely under heterogeneous unknown sensing delays and temporally correlated measurement noise. Can a bounded residual policy learn to compensate for these uncertainties while preserving the nominal controller?

"But time-delay systems are a well-studied control problem."

I'd say:

"Absolutely. I am not proposing that delay compensation itself is novel. My baseline is the existing decentralized cooperative controller, and the question is whether a bounded residual policy can improve robustness when each agent experiences an unknown, heterogeneous, time-varying sensing delay and temporally correlated measurement noise. The RL component does not replace the controller; it learns a corrective residual around it."

"Robust control provides an important theoretical baseline, but my objective is not to derive a worst-case delay-dependent stability condition. I am investigating whether a learned residual can exploit temporal information and the structured dynamics of the cooperative system to compensate for uncertainty that is difficult to model exactly."

"Classical delay-robust approaches exist, but the present work investigates a learning-based residual augmentation of an established controller.

unknown, heterogeneous, time-varying delays

Then the agents are operating on inconsistent information:

x1(t−τ1),x2(t−τ2),x3(t−τ3),x4(t−τ4).x_1(t-\tau_1),\quad x_2(t-\tau_2),\quad x_3(t-\tau_3),\quad x_4(t-\tau_4).

The problem isn't merely stabilizing a single delayed plant.

It's maintaining cooperative behavior when the agents have asynchronous, partially inconsistent observations of the shared state.

That's a substantially more interesting setting.

And you can make it even more explicit:

The uncertainty is not only in the physical dynamics; it is in the information available to each member of the cooperative system.

That is where residual learning has a plausible argument.

"I can tolerate 80 ms delay in this payload configuration, so don't behave as conservatively as the worst-case robust controller."

IL is constrained by the behavior of the teacher, whereas RL allows policy improvement with respect to objectives that are not encoded in the teacher's control law." Additionally, it’s still very lightweight (because optimal control can achieved also improvements in directions we want)

IL provides a lightweight approximation of the existing optimal controller, while residual RL enables further policy improvement in selected directions—such as robustness, energy efficiency, payload oscillation, or transient performance—without reintroducing the computational cost of online optimal control.

four controllers are simultaneously acting on a coupled physical system while each has a different, uncertain, temporally stale view of the shared state."

There isn't a magic

PID + low-pass filter\boxed{\text{PID + low-pass filter}}

that generally makes that disappear.

Mention failed kf, mention that  history adds nothing to it so far also, it's solution is not close at all to the clean controller, it learnt something very different (aka can't be trained with supervised ML)

Can a lightweight decentralized learned policy discover effective cooperative behavior under an information structure for which the nominal controller fails?

Suppose your professor says:

"Why don't you just use an optimal delay-aware estimator?"

Your honest answer is:

"That may be possible. My work does not claim that it is impossible or that RL is theoretically necessary. I'm investigating whether a learned policy can directly achieve robust decentralized behavior under the degraded information structure, without explicitly estimating or compensating for each agent's unknown delay."

That's completely legitimate.

If they say:

"But surely someone could design a classical solution."

Answer:

"Probably. The question I'm studying is whether learning can discover an effective policy without explicitly constructing that model-based solution. While also still being lightweight at deployment"

And that's the important distinction.

"We investigate whether a learned decentralized policy can provide robust cooperative payload control under heterogeneous asynchronous observations, while avoiding the computational cost of the original controller."

so i think there's a problem, the agent is not necessarily being well incentivized to avoid blowups, by avoiding blowups, the episode length increases but with the drones at high velocity and then still it eventually explodes, so the agent kinda get more punishment for surviving longer and doing actions that decreases blowups

solution, just train on a trajectory that doesn't blow up(at least the deterministic policy doesn't) , with some margin so the incentive structure show be better

The overspeed penalty accrues per step, and the blowup is a fixed -100 terminal. If every rollout the agent can currently produce eventually blows within the horizon, then total = (steps survived)·(overspeed/step) + (−100). Surviving longer only grows the first term while the second is fixed → early death is optimal, and the exploratory high-speed thrashing that's actually on the path to a stabilizing policy gets punished hardest.

The gradient literally points away from the messy intermediate behavior you need to pass through to reach survival.

The fix is to make a no-blowup outcome reachable with margin. The moment the task is feasible, the good policy flies sub-speed-3 → overspeed penalty ≈ 0 → survival is penalty-free and reaching the end avoids the -100. The incentive flips from "die early" to "survive and track."

The following observations were made

- training on pure y axis movement for 150 iterations did not meaningfully harm the x axis

- Training on the x axis and y axis had to be done separately, learning from one axis did not transfer meaningfully to the other

These observations lead us to conclude that the net is memorizing a separate policy across the different axis and is not learning to generalize (maybe it’s lacking depth?). However it still has the capacity(could be in the width) to learn multiple axes without them interfering with each other.

On learning each trajectory on its own and not being able to learn them together

By distillation using BC we were able to fit a net to both policies so the joint map is a function and a single net fits it easily — but that also refutes "definitely capacity." The thing that stopped the joint RL run is PPO optimization: policy-gradient can't find this representable solution because every off-basin exploration step blows up. The net can hold the answer; RL can't reach it by exploring.

So why not train a bunch of nets on a bunch of trajectories

here's my worry with BC, on two trajectories, it risks memorizing a lot, and warm starting RL from there doesn't achieve much because it doesn't encode the right thing, now if i have two nets that are good on a bunch of trajectories and then i mix them with BC , that's more like it

solution: train a net that works badly (but doesn’t blow up) on a mix of trajs

Feasibility: if all 5 survive at scale 5, the mix is already feasible and you just run it. No gentling needed — the incentive trap only arms when something actually blows, and nothing does. "Working but badly" (loop 0.6–0.98) is exactly the regime joint RL is supposed to pull down; there's positive experience on every direction, nothing to dodge. So MIX_SCALE=5.0 as-wired, warm-started from it starts in a basin that's correct only on those two sticks and has learned nothing generalizable to lean on. That's why it "doesn't encode the right thing." BC's value is proportional to how much structure the sources already carry: distilling two nets that each generalize over a bunch of trajectories merges two real regions of competence → the BC net inherits transferable structure, and that's a seed worth RL-ing from.

Which means the sequencing is: this mix run is how you earn a net that's good over a bunch of directions in the first place. BC isn't the tool for now — it's the tool for later, to fuse two already-broad nets (say one strong on an +x-family bunch, one on a +y/-z-family bunch) into one. Get the broad nets first via joint mix RL; hold BC for the merge.

critB — has a clear canonical paper

"An Empirical Model of Large-Batch Training" — Sam McCandlish, Jared Kaplan, Dario Amodei, and the OpenAI Dota team, 2018. arXiv:1812.06162. This is the reference for the gradient noise scale and critical batch size — the tr(Σ)/|G|² quantity and the two-batch estimator we implemented are straight from it. Cite this one directly for critB.

gcos — no single canonical paper; it's a derived/folklore diagnostic

Be careful here — I don't want you citing something that doesn't exist. Cross-iteration gradient cosine is not from one definitive paper. It's a diagnostic I proposed this session, and its grounding is:

Same SNR decomposition as McCandlish (1812.06162). gcos is essentially a normalized read of the same signal-vs-noise idea: for g = G + noise, E[cos(g_t, g_{t+1})] ≈ SNR/(1+SNR). So the honest citation for gcos is also McCandlish — it's the cosine expression of that paper's noise scale, not a separate result.

The closest named relative is the task-gradient version, not the cross-iteration one: "Gradient Surgery for Multi-Task Learning" (PCGrad) — Yu et al., 2020, arXiv:2001.06782 — uses cosine similarity between different tasks' gradients to define and detect conflict (negative cosine = conflicting). That's a within-batch, between-task cosine; ours is between-iteration on the full gradient. Related idea, different axis — cite it as "gradient cosine similarity has been used to diagnose conflict (PCGrad)," not as the source of our exact metric.

So for a thesis:

critB → McCandlish et al. 2018 (arXiv:1812.06162). Solid, direct.

CritB increases non linearly 1 : ~300k, 2: ~ 1M, 3: ~15M

gcos → frame it as your own diagnostic, grounded in the McCandlish SNR decomposition, with PCGrad (Yu et al. 2020, arXiv:2001.06782) as the precedent for gradient-cosine as a training diagnostic. Don't attribute it to a single paper — that would be inaccurate.

why is the gradient pointing in any consistent direction that doesn't improve the reward? i mean the gadient comes from the advantages that come from rewards and critic that is super accurate , does gcos ~0.5 change direction enough such that the gradient is basically rotating in circles? not complete disagreement but slight change each iter that leads to eventually circling in the same space

On-policy PG is not gradient descent on a fixed function. The advantage is computed under the current policy's state distribution. As θ moves, the state visitation and the reward-to-go both shift — you're chasing a moving target, seeking a fixed point of a self-referential map, not minimizing a static loss. Such maps routinely have rotational (non-conservative) dynamics near a non-attracting fixed point. A critic that perfectly fits this iter's distribution is fitting a target that moves next iter → orbit.

Multi-agent coupling. This is a differentiable game (per-drone advantages, shared net). The gradient field of a game decomposes into a potential part (descends) + a Hamiltonian/rotational part (circulates) — Balduzzi et al., The Mechanics of n-Player Differentiable Games, 2018. When the rotational part dominates locally, plain gradient steps spiral/cycle instead of converging. Cooperative + shared-net weakens it but the shared single-advantage credit assignment across 4 drones can still leave a curl.

Basically non stationary problem, each sample points in a slightly different direction, no matter how big it’s still a sample and that’s why the difference is not big, then it keeps circling around a local minimum, this action is slightly better here, that is slightly better there,, etc

Useful results:

scale test  ckpt=residual_mappo.pt  dir=(1.0, -1.0, 0.3)  desync=ON  (mean over episode, GRACE-skipped)

move(m)  mode        loop    load  sat_lam   sat_w

7.0      base       1.375   0.417     0.00    0.00

policy     0.365   0.078     1.10    0.50

history doesn’t help, so no estimation is happening probably.